TEK-BOT IR REMOTE CONTROL TROUBLE SHOOTING GUIDE

| The LED does not glow | IS IT ON? Otherwise check the battery voltage. The transmitter should be above 8.0V and the receiver above 6.0V. Replace transmitter battery or charge tek-bot battery. |
| --- | --- |
| The LED glows RED | The power indicator LED should glow GREEN. If the LED glows RED the positive and negative power terminals are reversed. Simply reverse the power wire and the LED should again glow green. |
| The tek-bot will only move in one direction | The DR, ER, DL, DR outputs of the receiver board must be properly connected to the motor controller board. Make certain these pins line up correctly on each board. |
| The tek-bot keeps moving after I release a button | Make certain J3 (jumper 3) is in place. This toggles the receiver between momentary and latch mode. |
| The LED is GREEN but it doesn’t work | Check to be certain all of the correct components are in place. Recheck component values and orientation to be certain all components are correctly installed. Pay special attention to the orientation of the diode and the IC |
| The LED is GREEN but it doesn’t work | The address DIP-switches on the transmitter and receiver must be set to the same address. Make certain the switches are set exactly the same. |
| The LED is GREEN but it doesn’t work | Jumper pins J2 must be connected to the IR and center pin. If the jumper is on the RF to center pin the receiver will not function. |
| The LED is GREEN but it doesn’t work | Another common problem is “cold solder joints”. This means that the heat from the solder iron was not applied for a long enough time to efficiently saturate the connection with solder. The solution is to go back over every connection and re-solder every joint applying heat until the joint is fully saturated |
| The LED is GREEN but it doesn’t work | Make certain all pins of the IR detector are in the socket. It is very common for the center pin to miss the socket. |
| The LED is GREEN but it doesn’t work | Check that the motor control board had power. Use a multi-meter to test the voltage at the supply pins. |

Still having problems?  Contact: 
Ken Townsend
(402) 554-2927
ktownsend@mail.unomaha.edu

Ben Barenz

(402) 554-

bbarenz@mail.unomaha.edu