Components for Control Board

Dashed outlines mean component orientation is critical

3:  R5, R16, R17

330 Ω Resistor

Orange-Orange-Brown

4:  R2, R9, R14, R20

1k Ohm Resistor

Brown-Black-Red

5:  R4, R6, R12, R13, R15

10k Ohm Resistor

Brown-Black-Orange

1:  R10

33k Ohm Resistor

Orange-Orange-Orange

4:  R3, R7, R11, R18

100k Ohm Resistor

Brown-Black-Yellow

1:  R8

51k Ohm Resistor

Green-Brown-Orange

2: Uc_A, Uc_B

28-Pin Dip Socket

ATMEGA_48

1: R19

220K Ohm Resistor

Red-Red-Yellow

20: D0-D3, D5-D20

1N4148 Diode

Match diode stripe with stripe on board

1:  S1

SPST Switch.  Pin spacing allows only two ways to place.  Either way is OK.

1:  Uc_C

8-Pin Dip Socket

Attiny 25

1: 3-Pin Female Fuse Socket

IC Select

SPDT Slide Switch

2: LED_1, LED_2

Bi-Color

Longer lead is +, Shorter is flat side

2:  3-Pin Male Header

ISP

1: LED_3

Red

Longer lead is +, Shorter is flat side

Q2, Q3:  C5019

Match shape with silkscreen

Q4:  MPSA65

Match shape with silkscreen

Control Board Components (cont)

4:  C1, C2, C4, C5

0.1μF Capacitor

2:  4-pin male connector

Left Motor, Right Motor

Plastic lip matches with stripe on board

1: D4

1N5402 Diode

Match diode stripe with stripe on board

1:  C3

200μF Capacitor

Long lead is +

1:  5V Regulator

Flat side against board

1:  Audio Transducer

+ on case matches + on board

1: 20-Pin Male Ribbon Cable Connector

Single slot to center of board

1: SPDT Relay

Only fits one way

1:  R1

1 Ω Resistor

“U” shaped wire.  Does not look like a resistor

1: Male Connector

Charger

Red wire is +

1: Female Connector

Battery

Red wire is +

______________________________________________________________________________

1: Q1.  TIP-127FP

Place flat side to outside of board.  Warning! Mount on Back Side of Board

2: H Bridge

L298

Warning! Mount on Back Side of Board

______________________________________________________________________________

2: μC_A, : μC_B : ATMEGA 48 Place in socket when board is completed.

Do Not Solder

1: μC_C : Attiny Place in socket when board is completed.

Do Not Solder

1: Fuse

Place in socket when board is completed.

Do Not Solder

1: 10-Pin Female Header*

Remote Controller Port

1: 3-Pin Female Header. IR RECEIVER  Use the 3 units left from the 6 unit strip. Don’t use the long female strip

2: 20-Pin Male Ribbon Cable Connector**

See note below

1: Jumper

Place on Bump Bot Male pins when board is completed.

1: ATMEGA 48

Place in socket when board is completed.

Do Not Solder