Hook, Line, and CEENBoT

Programming a TI graphing calculator for the CEENBoT

Run a Program

1.	Press PRGM

2.	Under EXEC, select program

3.	Press ENTER twice

Edit a Program

1.	Press PRGM

2.	Right arrow to EDIT

3.	Down arrow to program name, press ENTER

4.	Edit the program

5.	Use 2nd MODE (QUIT) when finished

Program Editing Commands

In program editor, use keys and menus to enter commands.

CLEAR key - return to programming (if wrong key pressed)

2nd DEL (INS) - turn on insert mode

PRGM key

CTL: If..Then..Else..End

I/O: Disp, Input, Prompt, Get, Send

EXEC: use to call other programs as subroutines

MATH key

NUM: abs, int

PRB: rand

2nd [1] (L1)

Programs

TURNDEMO: Turn a random angle		TURNTEST: Forward 750 steps, turn 750 steps, forward again

TURNDEMO	TURNTEST

{3,0,0,0,0}->L1	{10,500}->L1:Send(L1):Get(Z)

int(rand*500+100)->L1(3)	{2,2,0,750,200,0}->L1:Send(L1):Get(Z)

int(rand*350+50)->L1(4) 	{3,0,750,200,0}->L1:Send(L1):Get(Z)

Send(L1)	{2,2,0,750,200,0}->L1:Send(L1):Get(Z)

Get(Z)

TI Calculator Commands to Control the CEENBoT

Step & Wait	{2, <which>,<dir>,<steps>,<speed>,<brk_mode>}

Tank Turn	{3, <which_way>,<steps>,<speed>,<brk_mode>}

Delay	{10, <delay_ms>}

Which	0 = left motor	1 = right motor	2 = both motors

Dir	0 = forward	1 = reverse

Steps	1 revolution = 	200 steps based on the motor design

Speed	0 to 400 		in steps/sec

Which Way	0 = left turn	1 = right turn

Brk Mode	0 = off		1 = on