Button Sequence for Programming the TI Robot

Programming the TI-Bot:

Every program command includes four numbers.  The first three numbers are all single digit numbers that provide basic commands to the servo-motors and the bumper sensors on the front of the robot.  The fourth number represents a time and can range from 1-10000.  The syntax and description of the programming is illustrated below.

The syntax of each programming command looks like the following:

Send ( {# # # , # } )

Get (R)

The “Send” command sends instructions from the calculator to the robot.  The “Get” command retrieves information from the robot.   Every “Send” command must be accompanied by a “Get” command.

The First Number:  	1 – Forward for a specified time (the

specified time is the 4th number)

2 – Forward until the bumper is

depressed

3 – Forward for a specified time or

until the bumper is depressed

The Second Number:	0 – Left motor in reverse

1 – Left motor inactive

2 – Left motor forward

The Third Number:	0 – Right motor in reverse

1 – Right motor inactive

2 – Right motor forward

The Fourth Number:	Time in centiseconds.   For example:			300 is three seconds.

Example:  This program tells the robot to go forward for three seconds, turn right, and go forward for 5 seconds or until it hits something.

Send ({122,300})   Both motors going forward for three seconds

Get (R)

Send ({120,50})     Left motor going forward, right motor going

Get (R)                backward for ½ of a second

Send ({322,500})   Both motors going forward for five seconds

Get (R)                    or until the bumper is depressed