The TI–Bot Users Guide

Specifications/Justifications:

The TI-robot from Norland research (www.tirobot.com) is a simple programmable robot that interfaces with the TI-83+ and TI-84+ series graphing calculators.  The programming of the robot is done via the calculator keyboard in the PROGRAM mode.  There are also a number of downloadable programs from the website and from the included CD, which can be directly loaded into the graphing calculator from the computer (no need to punch them in by hand).  The programming commands and syntax are simple enough to be used by middle school students and will help expose these students to innovative uses of the graphing calculator long before they would ordinarily have the opportunity to see one.  Because graphing calculators are now allowed on SAT and ACT exams, students who are not well versed in the use of graphing calculator technology are at a distinct disadvantage on these tests.  The TI-bot gives teachers the opportunity to involve students in 21st century thinking projects using mutually supportive 21st century technologies.  Have fun operating your TI-bot!!

Programming the TI-bot:

Every program command includes four numbers.  The first three numbers are all single digit numbers that provide basic commands to the servo-motors and the bumper sensors on the front of the robot.  The fourth number represents a time and can range from 1-10000.  The syntax and description of the programming is illustrated below.

The syntax of each programming command looks like the following:

Send ( {# # # , # } )

Get (R)

The “Send” command sends instructions from the calculator to the robot.  The “Get” command retrieves information from the robot.   Every “Send” command must be accompanied by a “Get” command.

The First Number:  	1 – Forward for a specified time (the specified time is the fourth number)

2 – Forward until the bumper is depressed

3 – Forward for a specified time or until the bumper is depressed

The Second Number:	0 – Left motor in reverse

1 – Left motor inactive

2 – Left motor forward

The Third Number:	0 – Right motor in reverse

1 – Right motor inactive

2 – Right motor forward

The Fourth Number:	Time in centiseconds.   For example 300 is three seconds.

Example:  This program tells the robot to go forward for three seconds, turn right, and go forward for 5 seconds or until it hits something.

Send ({122,300}) 	(both motors going forward for three seconds)

Get (R)

Send ({120,50})	(left motor going forward, right motor going backward for ½ of a second)

Get (R)

Send ({322,500})	(both motors going forward for five seconds or until the bumper is depressed)

Get (R)

TI-83+/84+ Button Pushing Sequence:

Task:  Program TI-bot to drive forward three seconds and display the time it traveled.

Step		Button		Button Location		Command

1.		   ON		Lower left side next to the 0	Power on

2.		PRGM		Three keys up from the 8		Enter program mode

3.			  (x2)	Right above the CLEAR key	Scroll to NEW for new program

4.	              ENTER		Bottom right			Create new program

5.		   2nd 		Yellow key

6.		ALPHA		Green key			Letters for the program name

7.		  Math		Below green key			Enter the letter A

8.		 APPS		Next to MATH key		Enter the letter B

9.		 PRGM		Next to APPS key		Enter the letter C

10. 		ENTER		Bottom right			Accepts the name

11.		PRGM		Three keys up from the 8		Get to the program menu

12.				Right above the CLEAR key	Get to the I/O Command menu

13.				Right above the CLEAR key	Get to the SEND command

14.		ENTER		Bottom right			Enter the SEND command

15.		   2nd		Yellow key

16.		    (		Right above the 8			Enter a left brace

17.		     1		1 key				Timed command

18.		     2		2 key				Left motor forward

19.		     2		2 key				Right motor forward

20.		     ,		Right above the 7 		Separates the commands

21.		   300		Enter the number 300		Three second time for the motors

22.		    2nd		Yellow key

23.		     )		Right above the 9			Enter a right brace

24.		     )		Same key as above		Enter a right parenthesis

25.		ENTER		Bottom right			Accepts the SEND command

26.		PRGM		Three keys above the 8		Starting to enter a GET command

27.				Above the CLEAR key		Get to the I/O command menu

28.			  (x2)	Above the CLEAR key		Get to the GET command

29.	              ENTER	  	Bottom right			Enter the GET command

30.	             ALPHA		Green key			Ready to enter a letter

31.		    X		Multiply key			Enter the letter “R”

32.		     )		Above the 9			Close right parenthesis

33.	              ENTER		Bottom right			Accept the GET command

34.		PRGM		Three keys above the 8		Starting to enter a DISP command

35.				Above the CLEAR key		Get to the I/O command menu

36.			  (x2)	Above the CLEAR key		Get to the DISP command

37.	              ENTER	  	Bottom right			Enter the DISP command

38.		      (		Above the 8			Open left Parenthesis

39.	             ALPHA		Green key			Ready to enter a letter

40.		    X		Multiply key			Enter the letter “R”

41.		     )		Above the 9			Close right parenthesis

34.		   2nd		Yellow key			Getting ready to leave the menu

35.		MODE		To the right of the yellow key	Quitting the PRGM menu

36.		PRGM		Three keys above the 8		Reentering the program mode

37.	              ENTER    (x2)	Bottom right			EXECuting the program

NOTE:  BEFORE YOU HIT THE ENTER KEY FOR THE LAST TIME, MAKE SURE YOUR ROBOT IS ON.  THE ON BUTTON IS ON THE BOTTOM OF THE TI-BOT.